109 _ TASK_SCHEDULER

# Concept 1️ - MSP vs PSP (Two Stack Pointers)

## First: What is a stack? (quick recap)

Stack is used to store:

- Local variables

- Function calls

- CPU registers during interrupts

👉 Cortex-M uses stack heavily for context switching

## Cortex-M has TWO stack pointers

| Stack Pointer | Full Form | Used by |
| --- | --- | --- |
| MSP | Main Stack Pointer | System / Interrupts |
| PSP | Process Stack Pointer | Tasks |

## MSP (Main Stack Pointer)

### Used by:

- Handler mode (ALL interrupts)

- Reset handler

- System code

👉 Default after reset:

CPU uses MSP

### Example

When interrupt occurs:

void SysTick_Handler(void)

{

// uses MSP

}

CPU automatically:

- Pushes registers to MSP

- Executes ISR

## PSP (Process Stack Pointer)

### Used by:

- Thread mode (tasks)

👉 Each task uses its own PSP

### Example

task1_handler();

If PSP is active:

- Task runs using its own stack

- Independent from other tasks

## Why TWO stack pointers?

This is the MOST IMPORTANT question.

### Problem if only one stack existed

- All tasks share same stack

- Interrupts also use same stack

- Data gets corrupted

- No task isolation

### Solution

| Stack | Purpose |
| --- | --- |
| MSP | System + interrupts |
| PSP | Individual tasks |

## Visual understanding

MSP (System Stack)

-------------------

| ISR data        |

| Scheduler data  |

-------------------

PSP (Task 1)

-------------------

| Task1 variables |

-------------------

PSP (Task 2)

-------------------

| Task2 variables |

-------------------

👉 Each task = separate stack memory

## VERY IMPORTANT RULE

Handler mode ALWAYS uses MSP

Even if PSP is active:

- Interrupt → uses MSP

## Switching from MSP → PSP

Your code:

switch_sp_to_psp();

Inside:

MSR PSP, R0

MSR CONTROL, #2

Meaning:

- Load PSP

- Tell CPU → “use PSP in Thread mode”

## CONTROL register connection (you learned before)

| Bit | Meaning |
| --- | --- |
| Bit 1 (SPSEL) | 0 = MSP, 1 = PSP |

So:

CONTROL = 0x02

👉 Thread mode now uses PSP

## Real-world analogy

- MSP → OS kernel stack

- PSP → Each app’s private stack

## Why this is important for scheduling

Because:

- Each task must have independent memory

- Scheduler switches PSP values

- That’s how CPU “jumps” between tasks

## Lock this concept

You should now clearly understand:

✔ Why two stack pointers exist
✔ MSP = system, PSP = tasks
✔ Interrupts always use MSP
✔ Tasks use PSP
✔ Switching PSP = switching task context

## Final clean understanding

You can now say:

“The CONTROL register is used to select PSP as the active stack for Thread mode, while MSP continues to be used for Handler mode. This enables separate stacks for tasks and system-level operations, which is essential for task scheduling.”

# Concept 2 — Task and Stack Allocation

This is one of the core ideas behind task scheduling.
If you understand this properly, the rest (PendSV, context switching) becomes much easier.

## 1. What is a Task?

In your code:

void task1_handler(void)

{

while(1)

{

printf("This is task 1\n");

}

}

Each such function is a task.

### Key point:

A task is simply a function that runs independently and has its own execution context.

## 2. What is “Task Context”?

A task context includes:

- CPU registers (R0–R12, LR, PC, xPSR)

- Stack contents

- Current execution point

### Important:

The CPU must save and restore this context to switch between tasks.

## 3. Why each task needs its own stack

### Problem (if single stack is used)

If all tasks use the same stack:

- Data gets overwritten

- Return addresses get corrupted

- Tasks interfere with each other

### Solution:

Each task gets its own dedicated stack region in SRAM

## 4. Stack allocation in your code

From main.h:

#define SIZE_TASK_STACK 1024U

#define T1_STACK_START (SRAM_END)

#define T2_STACK_START (SRAM_END - (1 * SIZE_TASK_STACK))

#define T3_STACK_START (SRAM_END - (2 * SIZE_TASK_STACK))

#define T4_STACK_START (SRAM_END - (3 * SIZE_TASK_STACK))

### Meaning:

You are manually dividing SRAM:

|-----------------------|  <-- SRAM_END

| Task 1 Stack          |

|-----------------------|

| Task 2 Stack          |

|-----------------------|

| Task 3 Stack          |

|-----------------------|

| Task 4 Stack          |

|-----------------------|

| Scheduler Stack (MSP) |

|-----------------------|

## 5. PSP storage per task

uint32_t psp_of_tasks[MAX_TASKS] = {

T1_STACK_START,

T2_STACK_START,

T3_STACK_START,

T4_STACK_START

};

### Meaning:

Each task has:

- Its own stack pointer value

- Stored in this array

## 6. Why we manually initialize stacks

This is the most important part.

Normally:

- When an interrupt occurs, hardware pushes registers onto stack

But here:

Tasks are not started by hardware interrupts

So we must manually simulate that stack frame.

## 7. What init_tasks_stack() is doing (concept only)

For each task:

- Move stack pointer down

- Push values as if CPU saved them

- Prepare stack so CPU can “resume” the task

## 8. What gets stored in each task stack

For every task, you create a fake stack frame:

xPSR  (Program status register)

PC    (Task function address)

LR    (Return behavior)

R12

R3

R2

R1

R0

R11

R10

R9

R8

R7

R6

R5

R4

### Important:

This matches exactly how Cortex-M saves context during an exception

## 9. Why this is needed

When scheduler switches to a task:

- It loads PSP

- CPU pops registers from stack

- Execution continues from PC

So:

Stack = complete snapshot of task

## 10. Key idea to remember

Task switching does NOT switch functions directly
It switches stack pointers

Changing PSP → changes task

## 11. One-line summary

Each task is given its own stack in SRAM, and its initial CPU context is manually created on that stack so the processor can start or resume execution as if it was previously running.

Each task is implemented as a separate function and is assigned its own dedicated stack in SRAM. Task scheduling is achieved by switching the Process Stack Pointer (PSP), which effectively switches the execution context between tasks. Although tasks appear to run in parallel, they actually execute sequentially using time slicing on a single-core processor. During context switching, the CPU register values (R0–R12, LR, PC, xPSR) are saved to and restored from the task’s stack in SRAM.

# Concept 3 — SysTick (Time Base for Scheduling)

## 1. What is SysTick?

SysTick is a built-in timer in Cortex-M processors used to generate periodic interrupts.

It is part of the CPU (not a peripheral like TIM2).

## 2. Why do we need SysTick in scheduling?

Recall:

CPU can run only one task at a time.

So we need a mechanism to say:

“After some time, switch to another task”

That “some time” is provided by SysTick.

## 3. Key Idea

SysTick acts as the heartbeat of the scheduler

Every tick:

- Interrupt occurs

- Scheduler gets control

- Decision: switch task or not

## 4. How SysTick works (conceptually)

SysTick is a down counter:

- Load a value (e.g., for 1 ms)

- Counter starts decrementing

- When it reaches 0:

  - Generates interrupt

  - Reloads automatically

  - Starts again

## 5. Flow in your system

Task running (Thread mode)

|

|  (after 1 ms)

V

SysTick interrupt occurs

|

V

CPU enters Handler mode

|

V

Scheduler logic runs

|

V

(Optional) trigger PendSV

|

V

Task switch happens

## 6. What your code is doing

init_systick_timer(TICK_HZ);

Where:

#define TICK_HZ 1000U

Meaning:

Generate interrupt every 1 ms

## 7. Important registers (concept level)

| Register | Purpose |
| --- | --- |
| SYST_RVR (0xE000E014) | Reload value |
| SYST_CSR (0xE000E010) | Control register |

## 8. What you configured

*pSRVR |= count_value;

This sets:

How long before interrupt occurs

*pSCSR |= (1<<1);

Enables:

SysTick interrupt

*pSCSR |= (1<<2);

Clock source:

CPU clock (16 MHz in your case)

*pSCSR |= (1<<0);

Starts:

SysTick counter

## 9. What happens every tick

Every 1 ms:

void SysTick_Handler(void)

{

// currently empty

}

But in a scheduler, this will:

- Decide next task

- Trigger context switch (via PendSV)

## 10. Important concept: Time Slicing

Each task gets CPU for a fixed time slice (e.g., 1 ms)

Example:

Time →

| T1 | T2 | T3 | T4 | T1 | T2 | ...

This is controlled by SysTick.

## 11. Why SysTick is used (instead of normal timers)

- Built into CPU

- Always available

- Low overhead

- Designed for OS/scheduler

## 12. Key connection to PendSV

Very important:

SysTick does NOT switch tasks directly

Instead:

- SysTick decides → “Switch needed”

- Then triggers → PendSV

- PendSV performs switch

## 13. One-line summary

SysTick is a periodic interrupt source that provides the time base for task scheduling and triggers the scheduler at fixed intervals.

## Checkpoint

You should now understand:

- Why scheduler needs a timer

- How SysTick generates periodic interrupts

- What happens every tick

- Why it is called “heartbeat”

- Why SysTick doesn’t directly switch tasks

# Step 1 — What is a Register (in this context)?

When you write:

uint32_t *pSCSR = (uint32_t*) 0xE000E010;

👉 You are pointing to a hardware register.

Think of it as:

A 32-bit box where each bit controls something.

## Example: SYST_CSR register (32 bits)

Bit position:

31 .............. 3  2   1   0

|  |   |   |

|  |   |   └── ENABLE

|  |   └────── TICKINT

|  └────────── CLKSOURCE

# Step 2 — What does (1 << n) mean?

This is the MOST IMPORTANT part.

## Example:

(1 << 0) → 0000 0001

(1 << 1) → 0000 0010

(1 << 2) → 0000 0100

So:

| Expression | Meaning |
| --- | --- |
| (1 << 0) | Bit 0 |
| (1 << 1) | Bit 1 |
| (1 << 2) | Bit 2 |

# Step 3 — What does |= mean?

*pSCSR |= (1 << 0);

Means:

“Set bit 0 to 1, keep everything else unchanged”

## Visual

Before:

0000 0000

After:

0000 0001

# Step 4 — Now apply to YOUR code

## 1. Enable SysTick counter

*pSCSR |= (1<<0);

### Meaning:

- Bit 0 = ENABLE

- Set to 1 → start timer

## 2. Enable SysTick interrupt

*pSCSR |= (1<<1);

### Meaning:

- Bit 1 = TICKINT

- Set to 1 → interrupt enabled

So when counter reaches 0:
→ interrupt will occur

## 3. Select clock source

*pSCSR |= (1<<2);

### Meaning:

- Bit 2 = CLKSOURCE

- 1 → CPU clock (16 MHz)

- 0 → external clock

# Step 5 — Reload register (SYST_RVR)

*pSRVR |= count_value;

This sets:

“After how many counts interrupt should occur”

## Example

If:

CPU clock = 16 MHz

TICK_HZ = 1000

Then:

count_value = 16000000 / 1000 - 1 = 15999

Meaning:

Count from 15999 → 0 → interrupt

# Step 6 — Why we clear first?

*pSRVR &= ~(0x00FFFFFFFF);

### Meaning:

- Clear old value

- Then write new value

## Breakdown

0x00FFFFFFFF → mask

~ → invert

&= → clear bits

# Step 7 — Full meaning of your configuration

Your code does:

- Set reload value → how long

- Enable interrupt → generate ISR

- Select clock → CPU clock

- Start counter → begin ticking

# Final simple understanding

You can say:

Each register is a 32-bit control unit where individual bits control specific features. Bit shifting (1 << n) is used to target a specific bit, and |= is used to set that bit without affecting others.

# One-line summary

(1 << n) selects a bit, and |= turns that bit ON inside a hardware register.

## Checkpoint

Make sure this is clear:

- What (1 << n) does

- What |= does

- What each SysTick bit controls

# Concept 4 — PendSV (Actual Context Switching)

This is where everything comes together:

- MSP vs PSP

- Task stacks

- SysTick

- Context switching

## 1. What is PendSV (in scheduling context)?

PendSV is a special interrupt used to perform context switching between tasks.

Key property:

PendSV is designed to run at the lowest priority

## 2. Why PendSV is used for context switching

### Problem

When SysTick interrupt occurs:

- You are inside an ISR (Handler mode)

- Interrupts must be fast

- Context switching is heavy (many registers)

So:

You should NOT switch tasks inside SysTick

### Solution

SysTick does:

“Task switch needed”

PendSV does:

“Actually perform the switch”

## 3. Flow of scheduling

Task running (Thread mode, PSP)

|

| SysTick interrupt

V

SysTick_Handler (MSP)

|

| decide next task

| trigger PendSV

V

PendSV pending

|

V

PendSV_Handler runs (MSP)

|

| save current task context

| load next task context

V

Return to Thread mode (PSP)

|

V

Next task runs

## 4. What is “Context Switching”?

Context switching = saving current task state + restoring next task state

### What is saved?

Registers:

R0–R12

LR

PC

xPSR

## 5. Important hardware behavior

When interrupt occurs:

CPU automatically saves:

R0, R1, R2, R3, R12, LR, PC, xPSR

This is called:

Hardware stacking

### What software must save

Remaining registers:

R4–R11

This is done inside PendSV.

## 6. What PendSV actually does

### Step-by-step

### Step 1 — Save current task

- Get current PSP

- Push R4–R11 onto stack

- Store updated PSP

### Step 2 — Select next task

current_task = (current_task + 1) % MAX_TASKS;

### Step 3 — Load next task

- Get PSP of next task

- Pop R4–R11

- Set PSP

### Step 4 — Exit PendSV

CPU automatically restores:

R0–R3, R12, LR, PC, xPSR

Now:

Next task starts executing

## 7. Why PendSV must be lowest priority

Very important:

Context switching must not interrupt other interrupts

So:

- All high-priority interrupts finish first

- Then PendSV runs

## 8. Key insight

Switching PSP = switching task
Stack = full task state

## 9. Relation to your code

Right now in your code:

void SysTick_Handler(void)

{

// empty

}

Later, you will do:

// trigger PendSV

*(uint32_t*)0xE000ED04 |= (1 << 28);

This sets:

PendSV pending

## 10. One-line summary

PendSV is a low-priority exception used to safely perform context switching by saving and restoring task stack states.

## Final mental model

SysTick → decides

PendSV → executes

PSP → holds task state

MSP → used during switching

## Checkpoint

You should now clearly understand:

- Why PendSV exists

- Why it is separate from SysTick

- What context switching means

- What is saved/restored

- Why lowest priority is required

# Concept 5 — Complete Scheduling Flow (Putting Everything Together)

This will connect:

- MSP / PSP

- Task stacks

- SysTick

- PendSV

- Context switching

## 1. Initial State (After Reset)

- CPU starts in Thread mode

- Uses MSP

- Executes main()

## 2. System Setup Phase (your code)

In main():

### Step 1 — Enable faults

enable_processor_faults();

### Step 2 — Setup scheduler stack (MSP)

init_scheduler_stack(SCHED_STACK_START);

Meaning:

MSP is now reserved for system (interrupts + scheduler)

### Step 3 — Assign task handlers

task_handlers[i] = taskX_handler;

### Step 4 — Initialize task stacks

init_tasks_stack();

Meaning:

Each task now has a ready-to-run fake context

### Step 5 — Start SysTick

init_systick_timer(TICK_HZ);

Meaning:

Scheduler heartbeat started (e.g., 1 ms)

### Step 6 — Switch to PSP

switch_sp_to_psp();

Meaning:

Thread mode will now use task stack (PSP)

### Step 7 — Start first task

task1_handler();

## 3. Task Execution Phase

Now:

- CPU is in Thread mode

- Using PSP

- Running Task 1

## 4. SysTick Interrupt Occurs

After 1 ms:

CPU does:

- Switch to Handler mode

- Use MSP

- Enter:

SysTick_Handler()

## 5. Inside SysTick (scheduler decision)

In future, this will:

- Decide next task

- Trigger PendSV

Example:

SCB->ICSR |= (1 << 28);  // trigger PendSV

## 6. PendSV Execution (actual switching)

After SysTick exits:

PendSV runs:

### Step 1 — Save current task

- Save R4–R11

- Store PSP

### Step 2 — Select next task

current_task = (current_task + 1) % MAX_TASKS;

### Step 3 — Load next task

- Load PSP

- Restore R4–R11

### Step 4 — Exit PendSV

CPU automatically restores:

- R0–R3, R12, LR, PC, xPSR

## 7. New Task Starts

Now:

- CPU back to Thread mode

- Using PSP of new task

- New task runs

## 8. Continuous Loop

Task1 → SysTick → PendSV → Task2

Task2 → SysTick → PendSV → Task3

Task3 → SysTick → PendSV → Task4

Task4 → SysTick → PendSV → Task1

## 9. Complete Flow Diagram

main()

↓

Setup MSP, PSP, stacks

↓

Start Task1 (Thread mode, PSP)

↓

(SysTick interrupt)

↓

Handler mode (MSP)

↓

Trigger PendSV

↓

PendSV_Handler

↓

Context switch

↓

Next Task runs (Thread mode, PSP)

## 10. Key Roles Summary

| Component | Role |
| --- | --- |
| MSP | System stack (interrupts, scheduler) |
| PSP | Task stack |
| SysTick | Time trigger |
| PendSV | Context switching |
| Stack | Stores task state |

## 11. One-line summary

The scheduler uses SysTick to periodically trigger PendSV, which performs context switching by saving and restoring task contexts using PSP, while MSP is used for interrupt handling.

## Checkpoint

You should now have a complete mental picture of:

- How tasks are created

- How CPU switches between them

- Role of each component

# Code Walkthrough — Step 1 (System Setup in main())

Let’s focus only on this part:

int main(void)

{

enable_processor_faults();

init_scheduler_stack(SCHED_STACK_START);

task_handlers[0] = (uint32_t) task1_handler;

task_handlers[1] = (uint32_t) task2_handler;

task_handlers[2] = (uint32_t) task3_handler;

task_handlers[3] = (uint32_t) task4_handler;

init_tasks_stack();

init_systick_timer(TICK_HZ);

switch_sp_to_psp();

task1_handler();

for(;;);

}

## Step 1 — enable_processor_faults()

uint32_t *pSHCSR = (uint32_t *) 0xE000ED24;

*pSHCSR |= (1<<16); // mem manage

*pSHCSR |= (1<<17); // bus fault

*pSHCSR |= (1<<18); // usage fault

### Concept:

You are enabling:

- Memory faults

- Bus faults

- Usage faults

### Why?

To catch errors during development instead of silent failure

If something goes wrong:
→ You enter fault handlers and print debug message

## Step 2 — init_scheduler_stack(SCHED_STACK_START)

__asm volatile ("MSR MSP, %0": : "r" (sched_top_of_stack): );

### Concept:

You are setting:

MSP = Scheduler stack

### Why?

- Interrupts use MSP

- Scheduler (PendSV, SysTick) uses MSP

So:

MSP is reserved for system-level operations

## Step 3 — Assign task handlers

task_handlers[0] = (uint32_t) task1_handler;

### Concept:

You are storing:

Address of each task function

So later:

- Scheduler knows where to jump

## Step 4 — init_tasks_stack()

This is VERY important (we will deep dive next step).

### Concept:

You are:

Creating fake initial stack frame for each task

So CPU can:

“Resume” task as if it was already running

## Step 5 — init_systick_timer(TICK_HZ)

### Concept:

You start SysTick:

- Generates interrupt every 1 ms

- Will later drive scheduling

## Step 6 — switch_sp_to_psp()

### Concept:

You are telling CPU:

“From now on, Thread mode uses PSP (task stack)”

So:

- Tasks → PSP

- Interrupts → MSP

## Step 7 — Start first task

task1_handler();

### Important:

This is NOT real scheduling yet.

You are:

Manually starting Task 1

Later:

- Scheduler will switch tasks automatically

## Step 8 — Infinite loop

for(;;);

Not used because:

- Task 1 already has infinite loop

# Summary of Step 1

Your main() is doing:

- Enable fault handling

- Setup MSP (scheduler stack)

- Store task function addresses

- Prepare task stacks

- Start SysTick timer

- Switch to PSP

- Start first task

## Key insight

main() only does setup. Real scheduling happens later via interrupts.

## Checkpoint

You should now clearly understand:

- Why MSP is configured

- Why PSP is enabled

- Why task handlers are stored

- What role SysTick plays

# Code Walkthrough — Step 2

# init_tasks_stack() (Core of Task Scheduling)

## First: What is this function doing?

It manually creates a fake stack frame for each task so that the CPU can start executing it as if it was already running.

## The function

void init_tasks_stack(void)

{

uint32_t *pPSP;

for (int i=0; i<MAX_TASKS; i++)

{

pPSP = (uint32_t *)psp_of_tasks[i];

pPSP--;

*pPSP = DUMMY_XPSR;

pPSP--; // PC

*pPSP = task_handlers[i];

pPSP--; // LR

*pPSP = 0xFFFFFFFD;

for(int j=0; j<13; j++)

{

pPSP--;

*pPSP = 0;

}

psp_of_tasks[i] = (uint32_t) pPSP;

}

}

# Step-by-step breakdown

## Step 1 — Get stack start

pPSP = (uint32_t *)psp_of_tasks[i];

### Meaning:

- Get top of stack for task i

- Stack grows downwards

## Step 2 — Push xPSR

pPSP--;

*pPSP = DUMMY_XPSR;

### What is xPSR?

- Program Status Register

- Bit 24 (T-bit) must be 1 for Thumb mode

#define DUMMY_XPSR 0x01000000U

### Meaning:

“This task will execute in Thumb mode”

## Step 3 — Push PC (Program Counter)

pPSP--;

*pPSP = task_handlers[i];

### Meaning:

When this task starts, execution begins at:

taskX_handler()

## Step 4 — Push LR

pPSP--;

*pPSP = 0xFFFFFFFD;

### VERY IMPORTANT

This value tells CPU:

- Return to Thread mode

- Use PSP

- Not MSP

This is called:

EXC_RETURN value

## Step 5 — Push remaining registers

for(int j=0; j<13; j++)

{

pPSP--;

*pPSP = 0;

}

This fills:

R0–R12

### Why 13?

Registers:

- R0–R12 → 13 registers

## Step 6 — Save updated PSP

psp_of_tasks[i] = (uint32_t) pPSP;

### Meaning:

Store the new stack pointer position

This is the point from where:

- Context will be restored

# Final stack structure (VERY IMPORTANT)

After initialization, stack looks like:

Higher Address

-----------------------

Initial Stack Top

-----------------------

xPSR        (0x01000000)

PC          (task_handler)

LR          (0xFFFFFFFD)

R12

R3

R2

R1

R0

R11

R10

R9

R8

R7

R6

R5

R4

-----------------------

Lower Address (PSP)

# Why this works

When context switch happens:

- CPU loads PSP

- CPU pops registers automatically:

  - PC → jumps to task

  - xPSR → execution state

So:

Task starts running without calling it normally

# Key concept

You are simulating what hardware normally does during an interrupt

# Important insight

This is exactly how:

- FreeRTOS

- Zephyr

- All RTOS

start tasks internally

# One-line summary

init_tasks_stack() prepares each task’s stack with an initial CPU context so that the processor can start executing the task via context restoration instead of a direct function call.

# Checkpoint

You should clearly understand:

- Why xPSR is set

- Why PC = task handler

- Why LR = 0xFFFFFFFD

- Why registers are pre-filled

- Why PSP is updated

# Code Walkthrough — Step 3

# switch_sp_to_psp() and First Task Launch

This is where your system actually transitions from setup → task execution.

## Function

__attribute__((naked)) void switch_sp_to_psp(void)

{

__asm volatile ("PUSH {LR}");

__asm volatile ("BL get_psp_value");

__asm volatile ("MSR PSP, R0");

__asm volatile ("POP {LR}");

__asm volatile ("MOV R0, #0X02");

__asm volatile ("MSR CONTROL, R0");

__asm volatile ("BX LR");

}

# Step-by-step understanding

## Step 1 — Preserve LR

PUSH {LR}

### Why?

- This function is “naked” (no compiler-generated prologue/epilogue)

- So you must manually save return address

## Step 2 — Get PSP of current task

BL get_psp_value

uint32_t get_psp_value(void)

{

return psp_of_tasks[current_task];

}

### Meaning:

- current_task = 0

- So:

PSP = stack of Task 1

Returned value is in:

R0

## Step 3 — Load PSP

MSR PSP, R0

### Meaning:

Load Process Stack Pointer with Task 1 stack

## Step 4 — Restore LR

POP {LR}

## Step 5 — Switch to PSP

MOV R0, #0X02

MSR CONTROL, R0

### Meaning:

- CONTROL[1] = 1

- Thread mode will now use PSP

Important:

MSP is still used by interrupts

## Step 6 — Return

BX LR

### Meaning:

- Return to main()

- But now:

  - Thread mode uses PSP

# What just happened (very important)

Before this function:

Thread mode → MSP

After this function:

Thread mode → PSP (Task 1 stack)

# Now comes the important part

## In main():

task1_handler();

### What happens now?

- Task 1 starts running

- Uses its own stack (PSP)

- Infinite loop

# Important observation

At this point:

❌ No real scheduling yet
❌ No task switching yet

Only:

Task 1 is running continuously

# Where will switching come from?

From:

SysTick_Handler()

Currently:

void SysTick_Handler(void)

{

}

So:

Nothing happens yet

# What will be added later

Inside SysTick:

trigger PendSV

Inside PendSV:

- Save current task

- Load next task

- Switch PSP

# Key concept connection

This function is the bridge between:

System setup → Task execution

# One-line summary

switch_sp_to_psp() initializes the PSP with the first task’s stack and configures the processor so that Thread mode execution uses PSP instead of MSP.

# Final mental picture

main() using MSP

↓

switch_sp_to_psp()

↓

Thread mode switches to PSP

↓

Task 1 runs using its own stack

# Checkpoint

You should clearly understand:

- Why PSP is loaded

- Why CONTROL register is modified

- Why LR is saved/restored

- What happens after returning

So now the most important thing is:

Understanding the actual scheduler logic (SysTick + PendSV implementation)

## Let’s do this efficiently

👉 First, I will explain:

# What MUST be inside SysTick and PendSV (concept + expected logic)

Then:
👉 You can compare with your code
👉 If mismatch → you share your code and we fix it

# Step 4 — What is Missing (Actual Scheduler Logic)

## 1. SysTick Handler — Role

SysTick should NOT switch tasks directly.

It should:

void SysTick_Handler(void)

{

// 1. Trigger PendSV

}

## How to trigger PendSV

*(uint32_t*)0xE000ED04 |= (1 << 28);

This sets:

PendSV pending bit

## So final SysTick:

void SysTick_Handler(void)

{

*(uint32_t*)0xE000ED04 |= (1 << 28);

}

## 2. PendSV Handler — Role

This is the core scheduler

It must do:

### Step 1 — Save current task context

- Read PSP

- Push R4–R11

- Store PSP

### Step 2 — Select next task

current_task = (current_task + 1) % MAX_TASKS;

### Step 3 — Load next task context

- Load PSP

- Pop R4–R11

### Step 4 — Update PSP register

## Pseudo-flow of PendSV

Save current task (R4–R11)

Update PSP array

Switch task index

Load next task PSP

Restore registers

Return

## 3. Important: Why only R4–R11?

Because:

### Hardware already saves:

R0–R3, R12, LR, PC, xPSR

### So software saves:

R4–R11

## 4. What happens after PendSV finishes?

CPU automatically:

- Restores hardware-saved registers

- Starts executing new task

# Final Full Flow (Your System)

Task1 running

↓

SysTick interrupt

↓

SysTick_Handler

↓

PendSV triggered

↓

PendSV_Handler

↓

Save Task1

Load Task2

↓

Return to Thread mode

↓

Task2 runs

# What you should verify in your code

Check if your implementation has:

### In SysTick:

- PendSV trigger present

### In PendSV:

- Save R4–R11

- Update PSP

- Switch task index

- Restore R4–R11

# Overall Assessment

You have correctly implemented:

- Task stack initialization

- PSP switching

- Context save/restore

- Round-robin scheduling

- Fault handling

This is essentially a mini RTOS scheduler.

# Important Observation (Very Critical)

## You are doing context switching inside:

SysTick_Handler()

### This works, but:

This is NOT the recommended design in ARM Cortex-M

## Why this is a problem (conceptually)

Earlier we discussed:

- SysTick → should be lightweight

- PendSV → should handle context switching

But in your code:

__attribute__((naked)) void SysTick_Handler(void)

{

// full context switching happening here

}

So:

You combined SysTick + PendSV into one

## Why ARM recommends PendSV

Because:

- SysTick may interrupt other interrupts

- Context switching is heavy

- Can cause nested interrupt issues

## Correct architecture (ideal design)

SysTick_Handler:

trigger PendSV

PendSV_Handler:

do context switching

## So your design is:

| Feature | Status |
| --- | --- |
| Functional | Correct |
| Educational | Very good |
| Production design | Needs improvement |

# Now let’s review your SysTick_Handler

## Step-by-step (your implementation)

MRS R0, PSP

Correct:

Get current task stack pointer

STMDB R0!, {R4-R11}

Correct:

Save software-saved registers

PUSH {LR}

BL save_psp_value

Correct:

Save updated PSP

BL update_next_task

Correct:

Round-robin scheduling

BL get_psp_value

Correct:

Get next task PSP

LDMIA R0!, {R4-R11}

Correct:

Restore registers

MSR PSP, R0

Correct:

Update PSP

BX LR

Correct:

Exit ISR → hardware restores remaining registers

# Very Important Good Points in Your Code

### 1. Proper use of naked functions

You handled stack manually — correct.

### 2. Correct use of STMDB / LDMIA

This shows strong understanding of:

- Stack growth

- Register saving

### 3. Correct separation of helper functions

save_psp_value()

get_psp_value()

update_next_task()

Very clean design.

### 4. Correct use of modulo scheduling

current_task = current_task % MAX_TASKS;

# Minor Improvements (Recommended)

## 1. Move context switching to PendSV (important)

### Change SysTick to:

void SysTick_Handler(void)

{

*(uint32_t*)0xE000ED04 |= (1 << 28);

}

### Rename your current SysTick logic to:

void PendSV_Handler(void)

## 2. Priority configuration (advanced improvement)

PendSV should be lowest priority:

*(uint32_t*)0xE000ED20 |= (0xFF << 16); // PendSV priority

## 3. Add DSB/ISB (advanced, optional)

After switching CONTROL:

DSB

ISB

Ensures proper pipeline sync.

## 4. Avoid calling C functions inside naked ISR (important for future)

Currently:

BL save_psp_value

BL update_next_task

This works, but:

In real RTOS → everything is done in assembly for speed and safety

# Final Technical Verdict

### Your level right now:

You have reached:

Intermediate → Advanced Embedded Systems Level

Because you understand:

- Stack frames

- Context switching

- ARM exception model

- Scheduler basics

# One-line evaluation

This is a correct and functional cooperative scheduler implementation, but it should ideally separate scheduling trigger (SysTick) and context switching (PendSV) for proper ARM-compliant design.

Overview:

“In the ARM Cortex-M3/M4 course by FastBit Academy, I focused on understanding how the processor works at a low level, especially in the context of task scheduling and RTOS fundamentals.”

“I learned key concepts like Thread mode and Handler mode, privileged and unprivileged access levels, and how the processor uses two stack pointers — MSP for system-level operations and PSP for task-level execution.”

“I also understood how context switching works internally, where the processor saves and restores register states using the stack, and how exceptions like SysTick and PendSV are used for scheduling.”

“As a practical implementation, I built a simple round-robin scheduler on an STM32 Cortex-M4 board, where I manually initialized task stacks, configured SysTick for time slicing, and implemented context switching by saving and restoring registers.”

“This gave me a clear understanding of how RTOS works internally, beyond just using APIs.”

Short Version:

“I learned how ARM Cortex-M handles task scheduling at a low level, including MSP/PSP stack usage, exception handling, and context switching using SysTick and PendSV. I also implemented a basic scheduler on STM32 to understand how RTOS works internally.”

1. How does context switching work?

During context switching, the processor automatically saves part of the register state, and the remaining registers are saved manually. The scheduler then switches the stack pointer to the next task and restores its context, allowing execution to continue from where it left off

2. Do you have any project related to the course?

As part of the course, I implemented a simple task scheduler on an STM32 Cortex-M4 board. It was a learning-based implementation where I created four tasks and used SysTick to generate periodic interrupts, and performed context switching by saving and restoring task states using the PSP stack.

I developed a basic round-robin task scheduler with four tasks to understand RTOS internals. I manually initialized task stacks, configured SysTick for time slicing, and implemented context switching by switching PSP values and saving/restoring registers.

It’s more of a learning-based mini project focused on understanding low-level scheduling rather than a full application-level project.