109 _ TASK_SCHEDULER

# Concept 1️ - MSP vs PSP (Two Stack Pointers)

## First: What is a stack? (quick recap)

Stack is used to store:

- Local variables

- Function calls

- CPU registers during interrupts

👉 Cortex-M uses stack heavily for context switching

## Cortex-M has TWO stack pointers

| Stack Pointer | Full Form | Used by |
| --- | --- | --- |
| MSP | Main Stack Pointer | System / Interrupts |
| PSP | Process Stack Pointer | Tasks |

## MSP (Main Stack Pointer)

### Used by:

- Handler mode (ALL interrupts)

- Reset handler

- System code

👉 Default after reset:

CPU uses MSP

### Example

When interrupt occurs:

void SysTick_Handler(void)

{

// uses MSP

}

CPU automatically:

- Pushes registers to MSP

- Executes ISR

## PSP (Process Stack Pointer)

### Used by:

- Thread mode (tasks)

👉 Each task uses its own PSP

### Example

task1_handler();

If PSP is active:

- Task runs using its own stack

- Independent from other tasks

## Why TWO stack pointers?

This is the MOST IMPORTANT question.

### Problem if only one stack existed

- All tasks share same stack

- Interrupts also use same stack

- Data gets corrupted

- No task isolation

### Solution

| Stack | Purpose |
| --- | --- |
| MSP | System + interrupts |
| PSP | Individual tasks |

## Visual understanding

MSP (System Stack)

-------------------

| ISR data        |

| Scheduler data  |

-------------------

PSP (Task 1)

-------------------

| Task1 variables |

-------------------

PSP (Task 2)

-------------------

| Task2 variables |

-------------------

👉 Each task = separate stack memory

## VERY IMPORTANT RULE

Handler mode ALWAYS uses MSP

Even if PSP is active:

- Interrupt → uses MSP

## Switching from MSP → PSP

Your code:

switch_sp_to_psp();

Inside:

MSR PSP, R0

MSR CONTROL, #2

Meaning:

- Load PSP

- Tell CPU → “use PSP in Thread mode”

## CONTROL register connection (you learned before)

| Bit | Meaning |
| --- | --- |
| Bit 1 (SPSEL) | 0 = MSP, 1 = PSP |

So:

CONTROL = 0x02

👉 Thread mode now uses PSP

## Real-world analogy

- MSP → OS kernel stack

- PSP → Each app’s private stack

## Why this is important for scheduling

Because:

- Each task must have independent memory

- Scheduler switches PSP values

- That’s how CPU “jumps” between tasks

## Lock this concept

You should now clearly understand:

✔ Why two stack pointers exist
✔ MSP = system, PSP = tasks
✔ Interrupts always use MSP
✔ Tasks use PSP
✔ Switching PSP = switching task context

## Final clean understanding

You can now say:

“The CONTROL register is used to select PSP as the active stack for Thread mode, while MSP continues to be used for Handler mode. This enables separate stacks for tasks and system-level operations, which is essential for task scheduling.”

# Concept 2 — Task and Stack Allocation

This is one of the core ideas behind task scheduling.
If you understand this properly, the rest (PendSV, context switching) becomes much easier.

## 1. What is a Task?

In your code:

void task1_handler(void)

{

while(1)

{

printf("This is task 1\n");

}

}

Each such function is a task.

### Key point:

A task is simply a function that runs independently and has its own execution context.

## 2. What is “Task Context”?

A task context includes:

- CPU registers (R0–R12, LR, PC, xPSR)

- Stack contents

- Current execution point

### Important:

The CPU must save and restore this context to switch between tasks.

## 3. Why each task needs its own stack

### Problem (if single stack is used)

If all tasks use the same stack:

- Data gets overwritten

- Return addresses get corrupted

- Tasks interfere with each other

### Solution:

Each task gets its own dedicated stack region in SRAM

## 4. Stack allocation in your code

From main.h:

#define SIZE_TASK_STACK 1024U

#define T1_STACK_START (SRAM_END)

#define T2_STACK_START (SRAM_END - (1 * SIZE_TASK_STACK))

#define T3_STACK_START (SRAM_END - (2 * SIZE_TASK_STACK))

#define T4_STACK_START (SRAM_END - (3 * SIZE_TASK_STACK))

### Meaning:

You are manually dividing SRAM:

|-----------------------|  <-- SRAM_END

| Task 1 Stack          |

|-----------------------|

| Task 2 Stack          |

|-----------------------|

| Task 3 Stack          |

|-----------------------|

| Task 4 Stack          |

|-----------------------|

| Scheduler Stack (MSP) |

|-----------------------|

## 5. PSP storage per task

uint32_t psp_of_tasks[MAX_TASKS] = {

T1_STACK_START,

T2_STACK_START,

T3_STACK_START,

T4_STACK_START

};

### Meaning:

Each task has:

- Its own stack pointer value

- Stored in this array

## 6. Why we manually initialize stacks

This is the most important part.

Normally:

- When an interrupt occurs, hardware pushes registers onto stack

But here:

Tasks are not started by hardware interrupts

So we must manually simulate that stack frame.

## 7. What init_tasks_stack() is doing (concept only)

For each task:

- Move stack pointer down

- Push values as if CPU saved them

- Prepare stack so CPU can “resume” the task

## 8. What gets stored in each task stack

For every task, you create a fake stack frame:

xPSR  (Program status register)

PC    (Task function address)

LR    (Return behavior)

R12

R3

R2

R1

R0

R11

R10

R9

R8

R7

R6

R5

R4

### Important:

This matches exactly how Cortex-M saves context during an exception

## 9. Why this is needed

When scheduler switches to a task:

- It loads PSP

- CPU pops registers from stack

- Execution continues from PC

So:

Stack = complete snapshot of task

## 10. Key idea to remember

Task switching does NOT switch functions directly
It switches stack pointers

Changing PSP → changes task

## 11. One-line summary

Each task is given its own stack in SRAM, and its initial CPU context is manually created on that stack so the processor can start or resume execution as if it was previously running.

Each task is implemented as a separate function and is assigned its own dedicated stack in SRAM. Task scheduling is achieved by switching the Process Stack Pointer (PSP), which effectively switches the execution context between tasks. Although tasks appear to run in parallel, they actually execute sequentially using time slicing on a single-core processor. During context switching, the CPU register values (R0–R12, LR, PC, xPSR) are saved to and restored from the task’s stack in SRAM.